Lab Manual for Embedded System Design

Lab No. 2

Arduino Interfacing with Relay & Seven Segment

Display

Objectives

In this lab students are introduced to the interfacing of Arduino with Relay & seven segment display

# LAB # 2

Arduino Interfacing with Relay & Seven Segment Display

## Introduction

RELAY:

Relay is an electrically operated switch as shown in Fig. 2.1.

12V   relay  is   used   to   isolate   electrical   load   using

Fig. 2.1

:

Relay

.

Microcontrollers.  Relay  have  two  configurations.  i.e.  NO   (Normally Open) and NC (Normally Close). Relay have coil    which is energized by 12V that results a Switching Action   based on NO-NC Configuration. If Relay is NO configuration

then after the coil is Energized, Switching Action makes it in  NC State that connects the Load. The internal circuitry of 	 Relay is as shown in Fig. 2.2.

Fig. 2.2: Relay Internal Circuitry.

SEVEN SEGMENT DISPLAY:

A seven segment LED display is a special arrangement of 7 LED elements to form a rectangular shape using two vertical segments on each side with one horizontal segment on the top, middle, and bottom as shown in Fig. 2.3.

Fig 2.3: Seven segment display

By individually turning the segments on or off, numbers from 0 to 9 and some letters can be displayed. Seven segment displays sometime also have an eighth segment to display the decimal point. Therefore, a seven-segment display will require seven outputs to display a number, and one    more    output if the decimal point is to be displayed too. The segments are marked with non-capital letters: a, b, c, d, e, f, g and dot, where dot is the decimal point. The 8 LEDs inside the display can be arranged with a common    cathode or common anode configuration. With a common cathode display, the cathodes of the entire segment LEDs are tied together and this common point must be connected to the ground. A required LED segment is then turned on by applying logic 1 to its anode. In common anode displays, all the anodes are tied together and the common anode is connected to the supply voltage Vcc. Individual segments are turned on by applying logic to their cathodes. The pin configuration is shown as Fig. 2.4.

Fig. 2.4: Seven Segment Display (Types & Pin Configuration).

## Time Boxing

| Activity Name | Activity Time | Total Time |
| --- | --- | --- |
| Login Systems + Setting up Proteus and Arduino Environment | 3 mints + 5 mints | 8 mints |
| Walk through Theory & Tasks | 60 mints | 60 mints |
| Implement Tasks | 80 mints | 80 mints |
| Evaluation Time | 30 mints | 30 mints |
|  | Total Duration | 178 mints |

## Objectives

In this lab students are introduced to open source single board microcontroller-Arduino, and its various design applications in designing embedded systems. Digital hardware components like Relay and Seven Segment Display will be interfaced with it.

## Lab Tasks/Practical Work

1. Write a sketch to interface Arduino with the Relay. The Relay should be controlled  	by a SPDT Switch.

When the “Switch” is ON, “Relay” will be in “ON” state and Lamp will     glow, otherwise Lamp will not glow.

Connect and Switch with Digital Pin 13 and a Relay through Transistor to Digital Pin 3 of the Arduino.

Fig. 2.5: Arduino – Relay Connection

Code int buttonPin = 13; int relayPin = 3;

// Variables will change. int buttonState = 0;

void setup( )

{

pinMode(relayPin, OUTPUT); pinMode(buttonPin, INPUT);

}

void loop( )

{

buttonState = digitalRead(buttonPin);

// Check if the Push Button is Pressed. if (buttonState == HIGH)

{

digitalWrite(relayPin, HIGH);

}

else

{

digitalWrite(relayPin, LOW);

}

}

2. Write a sketch to interface Arduino with Seven Segment Display. It should work as a decade counter. The Start / Stop of counting should be controlled through a SPDT Switch.

Fig. 2.6

:

Arduino

–

Seven Segment Display Connection

Code:

| #define segA 2 #define segB 3 #define segC 4 #define segD 5 #define segE 6 #define segF 7 #define segG 8 #define button 10 | // Connecting Segment A to PIN 2. // Connecting Segment B to PIN 3. // Connecting Segment C to PIN 4. // Connecting Segment D to PIN 5. // Connecting Segment E to PIN 6. // Connecting Segment F to PIN 7. // Connecting Segment G to PIN 8. // Connecting SPDT Switch to PIN 10. |
| --- | --- |
| int COUNT=0; int ButtonState; | // Count Integer for 0-9 Increment. // Variable Checking the State of the Button. |

void setup()

{

for (int i=2;i<9;i++)

{

pinMode(i,OUTPUT); 	   // Taking all Pins from 2-8 as OUTPUT.

}

pinMode (10, INPUT);

} void loop()

{

ButtonState=digitalRead(button);

if(ButtonState==HIGH)

{

switch (COUNT)

{ case 0:

digitalWrite(segA, HIGH); digitalWrite(segB, HIGH); digitalWrite(segC, HIGH); digitalWrite(segD, HIGH);

digitalWrite(segE, HIGH);

digitalWrite(segF, HIGH);

digitalWrite(segG, LOW);

case 0:

digitalWrite(segA, HIGH); digitalWrite(segB, HIGH); digitalWrite(segC, HIGH); digitalWrite(segD, HIGH);

digitalWrite(segE, HIGH);

digitalWrite(segF, HIGH);

digitalWrite(segG, LOW);

break;

case 1:

digitalWrite(segA, LOW);

digitalWrite(segB, HIGH);

digitalWrite(segC, HIGH);

digitalWrite(segD, LOW);

digitalWrite(segE, LOW);

digitalWrite(segF, LOW);

digitalWrite(segG, LOW);

break;

case 2:                                                                                                                                                                     	digitalWrite(segA, HIGH);

digitalWrite(segB, HIGH); digitalWrite(segC, LOW); digitalWrite(segD,HIGH);

digitalWrite(segE, HIGH);

digitalWrite(segF, LOW);

digitalWrite(segG, HIGH);

break;

case 3:

digitalWrite(segA, HIGH);  	digitalWrite(segB, HIGH); digitalWrite(segC, HIGH); digitalWrite(segD, HIGH); digitalWrite(segE, LOW); digitalWrite(segF, LOW);  	digitalWrite(segG, HIGH); break;

case 4:

digitalWrite(segA, LOW); digitalWrite(segB, HIGH); digitalWrite(segC, HIGH);

digitalWrite(segD, LOW);

digitalWrite(segE, LOW);

digitalWrite(segF, HIGH); digitalWrite(segG, HIGH);

break;

case 5:

digitalWrite(segA, HIGH); digitalWrite(segB, LOW);  	digitalWrite(segC, HIGH); digitalWrite(segD, HIGH); digitalWrite(segE, LOW); digitalWrite(segF, HIGH); digitalWrite(segG, HIGH);

break;

case 6:

digitalWrite(segA, HIGH); digitalWrite(segB, LOW); digitalWrite(segC, HIGH); digitalWrite(segD, HIGH);

digitalWrite(segE, HIGH); digitalWrite(segF, HIGH); digitalWrite(segG, HIGH);

break;

case 7:

digitalWrite(segA, HIGH);  	digitalWrite(segB, HIGH);  	digitalWrite(segC, HIGH);

digitalWrite(segD, LOW); digitalWrite(segE, LOW);  	digitalWrite(segF, LOW); digitalWrite(segG, LOW);

break;

case 8:

digitalWrite(segA, HIGH);  	digitalWrite(segB, HIGH); digitalWrite(segC, HIGH); digitalWrite(segD, HIGH); digitalWrite(segE, HIGH); digitalWrite(segF, HIGH);  	digitalWrite(segG, HIGH); break;

case 9: digitalWrite(segA, HIGH); digitalWrite(segB, HIGH); digitalWrite(segC, HIGH); digitalWrite(segD, HIGH); digitalWrite(segE, LOW);  	digitalWrite(segF, HIGH); digitalWrite(segG, HIGH);

break;

}

if (COUNT<10)

{

COUNT++; delay(1000);

}

if (COUNT==10)

{

COUNT=0; delay(1000);

}

}